Multi Modal Intelligent Traffic Signal System

Simulation Deployment – User Manual

Revision 0.0

(Initial Release)

July 10, 2020

Table of Contents

1.	Purpose of Document									      3

2.	Requirements:										      3

3.	Simulation Model									      3

3.1. Configuring Connected Vehicle							      3

3.2. Signal Controller									      4

3.2.1. Configuring Econolite ASC/3 Signal Controller			      4

3.2.2. Configuring MaxTime Signal Controller				      6

4.	VISSIM Driver Model DLL								      7

5.	Simulation-Tools									      8

5.1. Message-Distributor								      9

5.2. Priority-Request-Generator-Server						     10

6.	Deployment – Docker Containers							     13

List of Figures:

Figure 1: Vissim simulation model of multiple intersections					         3

Figure 2: Defining Connected Vehicle Type							         4

Figure 3: Prepare to setup the parameters							         5

Figure 4: Define the parameters								         5

Figure 5: Starting MaxTime signal Controller						         6

Figure 6: Defining the IP address in MaxTime Signal Controller				         6

Figure 7: Flow diagram of Priority-Request-Generator-Server				        12

# Purpose of Document

This document is an instruction guide for deploying Multi-Modal Intelligent Traffic Signal System (MMITSS) applications in the simulation platform. The document contains the detailed configuration and usage instructions for deploying the MMITSS software components in the docker container.

# Requirements:

Following devices, software and libraries are required to be installed to deploy MMITSS applications in the simulation environment.

- VISSIM 10 or VISSIM 2020 installed in a windows computer

- A Linux box (preferably Ubuntu 18.04) installed with docker, docker-compose, supervisor

# Simulation Model

PTV Vissim (a microscopic simulation software) is used to simulate all modes of traffic and analyses the performance of MMITSS software components. Vissim can be utilized to create realistic and accurate in every detail to test different traffic scenarios. A sample Vissim model of DaisyMountain-GavilanPeak and DaisyMountain-DedicationTrail intersections at Anthem, Arizona is shown in figure1.

Figure 1: Vissim simulation model of multiple intersections

# 3.1. Configuring Connected Vehicle:

A new vehicle type (Connected vehicle) can be created using external driver model dll. In section 4, the method of setting up driver model dll is discussed.

- To create a connected vehicle say Transit, a new vehicle type required to be defined (figure2).

- After that, the directory of the diver model dll file has to define under the External Driver Model as shown in the figure 2.

- Finally this new vehicle type is required to assign under Base Data->Vehicle Classes.

Figure 2: Defining Connected Vehicle Type

3.2. Signal Controller:

To establish communication between the MMITSS software components signal controller, it is required to configure the signal controller. Different agencies use different type of signal controllers. In Tucson and Anthem use Econolite ASC/3 signal controller, Portland use MaxTime signal controller to control the traffic flow at each intersection.

3.2.1. Configuring Econolite ASC/3 signal controller:

Following steps are required to configure Econolite ASC/3 signal controller:

(a) At first define the IP address of the computer on which Vissim software. Mrp container and Vissim computer must be in the same subnet. For example- if the intersection container host IP address is: 10.254.56.XX then Vissim computer IP address will be 10.254.56.YY

(b) Next step is to define the controller IP, subnet mask, default gateway IP, server IP, NTCIP backup time and NTCIP UDP Port in the Vissim ASC/3 contoller. To do so, from the Vissim go to each signal controller and click “Parameters” option (figure3). Then go to the Comm-Pg1 option (MM 1-4) under configuration and define the parameters (figure4).

Figure 3: Prepare to setup the parameters

Figure 4: Define the parameters

3.2.2. Configuring MaxTime signal controller:

Following steps are required to configure MaxTime Signal controller:

(a) Start MaxTime signal controller by defining unique Web Port, PTV Vissim TCP port, PTV Vissim id for each intersection (figure 5).

Figure 5: Starting MaxTime signal Controller

(b) Then controller IP address has to define from peer configuration (figure 6).

Figure 6: Defining the IP address in MaxTime Signal Controller

# VISSIM Driver Model DLL

Built upon the original sample driver-model provided with VISSIM distribution, a custom driver-model has been developed to simulate connected vehicles that interact with MMITSS applications. Four variants of this driver-model are built which are the following:

- M_DriverModelPassenger.dll (type = “passenger”)

- M_DriverModelEmergency.dll (type = “emergency”)

- M_DriverModelTransit.dll (type = “transit”)

- M_DriverModelTruck.dll (type = “truck”)

At every time instant in the VISSIM simulation, vehicles using any of above driver-models send a UDP packet containing a Basic Safety Message (BSM) like JSON formatted information related to the vehicle’s identification, type, size, current location and motion to a network node defined in the configuration file. An example of JSON string developed by the driver model is as follows:

{

"MsgType": BSM,

"BasicVehicle": {

"heading_Degree": 359.1,

"position": {

"elevation_Meter": 587,

"latitude_DecimalDegree": 33.843455,

"longitude_DecimalDegree": -112.135159

},

"secMark_Second": 2600,

"size": {

"length_cm": 400,

"width_cm": 300

},

"speed_MeterPerSecond": 12.5,

"temporaryID": 5822735,

"type": "transit"

},

}

Functionally all four driver models are identical except that the value in the type field in the JSON string is different for each driver-model. These driver-models are tested in VISSIM 10 and VISSIM 2020.

To use any of these driver-models, a directory named “mmitss_simulation” needs to be placed in the same directory where the “*.inpx” file of the simulation exists. This directory is provided in the MMITSS simulation distribution package. The “mmitss_simulation” directory contains a configuration file named “mmitss_driver_model_config.json” and a subdirectory named “driver_models” where all four prebuilt driver-models are stored. All four driver-models use the same configuration file and it must be made sure that the relative location of the configuration file from the “*.inpx” file is not altered. The configuration file allows to configure the network node where the UDP packets of JSON strings need to be sent. In addition, it is required to input the GPS coordinates of the origin (0,0) of the simulation model in the configuration file. This information is required to correctly transform the vehicle’s current location from the VISSIM’s local coordinate system to the GPS coordinate system. An example of the content of the “mmitss_driver_model_config.json” is as follows:

{

"msg_distributor_ip": "127.0.0.1",

"msg_distributor_port": 5000,

"vissim_origin_position": {

"elevation_Meter": 540.11,

"latitude" : {

"Degree" : 33.0,

"Minute" : 50.0,

"Second" : 35.4

},

"longitude" : {

"Degree" : -112.0,

"Minute" : -8.0,

"Second" : -6.1

}

}

}

The IP address and UDP port of the receiver network node can be specified in the “msg-distributor-ip" and “msg-distributor-port" fields respectively. To interface with other MMITSS applications, this network-node must be the message-distributor component of MMITSS simulation-tools.

# Simulation-Tools

To interface simulation models with other MMITSS core components, two additional components are developed: message-distributor and priority-request-generator-server. The message-distributor component is responsible for receiving and distributing messages from VISSIM simulation model and other MMITSS components to the priority-request-generator-server component or to appropriate intersection containers, whereas the priority-request-generator-server component is responsible for formulating the Signal Request Messages (SRMs) based on the vehicle information received from the message-distributor component.

## 5.1. Message-Distributor

As the name suggests, the message-distributor component is responsible for receiving messages from VISSIM simulation model or other MMITSS components and distributing them to applicable configured clients. The message-distributor also allows to configure and simulate the wireless range (in Meter) of each intersection in the corridor along with a probability of dropping message packets. The clients (IP address and UDP port) for supported messages can be configured in the configuration file of the message-distributor. An example of the configuration file (mmitss-message-distributor-config.json) is as follows:

{

"package_drop_probability": 0,

"raw_bsm_logging": true,

"intersections": [

{

"name": "DaisyMountain_GavilanPeak",

"ip_address": "xxx.xxx.xxx.xxx",

"bsm_client_port": 5001,

"srm_client_port": 20002,

"dsrc_range_Meter": 500,

"position": {

"latitude_DecimalDegree": 33.842932,

"longitude_DecimalDegree": -112.135186,

"elevation_Meter": 539

}

},

{

"name": "DaisyMountain_DedicationTrail",

"ip_address": "xxx.xxx.xxx.yyy",

"bsm_client_port": 5001,

"srm_client_port": 20002,

"dsrc_range_Meter": 500,

"position": {

"latitude_DecimalDegree": 33.843239,

"longitude_DecimalDegree": -112.131541,

"elevation_Meter": 539

}

},

],

"bsm_clients":

{

"transit":[

{

"ip_address": "xxx.xxx.xxx.zzz",

"port":20022

}

],

"truck":[

{

"ip_address": "xxx.xxx.xxx.zzz",

"port":20022

}

],

"emergency":[

{

"ip_address": "xxx.xxx.xxx.zzz",

"port":20022

}

],

"passenger":[

]

},

"map_clients":

[

{

"ip_address": "xxx.xxx.xxx.zzz",

"port":20022

}

],

"ssm_clients":

[

{

"ip_address": "xxx.xxx.xxx.zzz",

"port":20022

}

]

}

The message-distributor receives and distributes following messages in the described way:

- BSM-like information from VISSIM simulation model:

If the vehicle type in the received message is either “emergency”, “transit”, or “truck”, the message is forwarded to the priority-request-generator-server component (or more clients if required).  In the configuration file above, the IP address xxx.xxx.xxx.zzz corresponds to the docker container (or physical machine) that hosts the priority-request-generator-server. In addition, regardless of the vehicle type, if the location described in the received message is within the wireless range (Meter) of any of the configured intersections, the message is distributed to that intersection’s BSM receiving client (defined by intersection_ip and bsm_client_port).

- SRMs from priority-request-generator-server

The priority-request-generator-server sends JSON strings of formulated SRMs to the message-distributor. If the location of vehicle (as per the received message) is within the wireless range of any (Meter) of any of configured intersections, the message is distributed to that intersection’s SRM receiving client (defined by intersection_ip and srm_client_port). For MMITSS priority applications, the SRM receiving client is the priority-request-server.

- SSMs from priority-request-server of each intersection

The priority-request-server component of each intersection that receives SRMs, generate and broadcast SSMs as an acknowledgement of the receipt of SRM if it is eligible for the priority service. The priority-request-server of each intersection also forwards such SSMs to the message-distributor, which then forwards these SSMs to configured SSM receiving clients. For MMITSS priority applications, one SSM receiving client is the priority-request-generator-server component.

- MAPs from map-spat-broadcasters of each intersection

The map-spat-broadcaster component of each intersection broadcasts MAP message at a frequency of 1 Hz. It also forwards the MAP messages to the message-distributor, which then forwards these MAPs to configured MAP receiving clients. For MMITSS priority applications, one MAP receiving client is the priority-request-generator-server component.

5.2. Priority-Request-Generator-Server

Priority-request-generator-server is responsible managing list of priority eligible vehicles. It receives BSMs from the message-distributor. If the vehicle ID is not available in the list, it creates an instance of priority-request-generator for that vehicle ID. On the other hand, if the vehicle ID is already listed, it updates the information in the list. Priority-request-generator-server also receives MAP messages from the message-distributor. It calculates the haversine distance between the map reference point and all vehicles current gps points. If the distance is within wireless range, it forwards the MAP messages to the vehicle. Each instance of priority-request-generator can generate SRM JSON string, based on the MAP and GPS points. The priority-request-generator-server sends those SRMs JSON strings to the message-distributor. It can also receive SSMs JSON strings form the message-distributor and forwards it to the vehicles priority-request-generator instances to maintain the Active Request Table (ART).

Figure 7: Flow diagram of Priority-Request-Generator-Server

# Deployment – Docker Containers

To deploy MMITSS in the simulation environment, following steps can be followed:

Step1: Create configuration file

It is required to create mmitss-phase3-master-config.json, mmitss-data-external-clients.json, configuration files for each intersection and mmitss-phase3-master-config.json, mmitss-message-distributor-config.json configuration files for simulation tools (message-distributor, priority-request-generator-server). Configuration files for each intersection need to be placed in the <intersectionName>/nojournal/bin directory and for simulation tools configuration files required to be placed in simulation-tools/nojournal/bin directory. An example of mmitss-phase3-master-config.json is follows:

{

"HostIp": "xxx.xxx.xxx.xxx",

"SourceDsrcDeviceIp": "xxx.xxx.xxx.yyy",

"IntersectionName": "xxx",

"MapPayload":001283fe38083020315abe2149d0eecf1800a0000271c4fcbd028280",

"IntersectionID" : XXXX,

"RegionalID" : 0,

"DataCollectorIP": "xxx.xxx.xxx.xyx",

"HMIControllerIP": "xxx.xxx.xxx.yxx",

"MessageDistributorIP": " xxx.xxx.xxx.zzz ",

"PriorityRequestGeneratorServerIP": "xxx.xxx.xxx.zzz",

"VehicleType" : 6,

"Logging" : "True",

"SRMTimedOutTime" : 10.0,

"PortNumber":

{

"MessageTransceiver":

{

"MessageSender": 10003,

"MessageReceiver": 10002,

"MessageEncoder": 10003,

"MessageDecoder": 10002

},

"MessageDistributor": 5000,

"RsmDecoder": 10006,

"OBUBSMReceiver": 10005,

"HostBsmDecoder": 10005,

"TrajectoryAware": 20001,

"PriorityRequestServer": 20002,

"PrioritySolver": 20003,

"PriorityRequestGenerator": 20004,

"TrafficControllerInterface": 20005,

"TrafficControllerCurrPhaseListener": 20006,

"TrafficControllerTimingPlanSender": 20007,

"PerformanceObserver": 20008,

"HMIController": 20009,

"PrioritySolverToTCIInterface": 20010,

"SignalCoordination": 20011,

"MapSPaTBroadcaster": 6053,

"DsrcImmediateForwarder": 1516,

"PriorityRequestServer_SendSSM": 50003,

"DataCollector": 30006,

"SnmpEngine": 20020,

"SnmpEngineInterface": 20021,

"PriorityRequestGeneratorServer": 20022

},

"psid":

{

"map": "E0000017",

"spat": "8002",

"rsm": "8003",

"srm": "E0000019",

"ssm": "E0000020",

"bsm": "20"

},

"msgId":

{

"map": "0012",

"spat": "0013",

"rsm": "0021",

"srm_lower": "001d",

"srm_upper": "001D",

"ssm_lower": "001e",

"ssm_upper": "001E",

"bsm": "0014"

},

"SignalController":

{

"IpAddress": " xxx.xxx.xxx.yyy",

"NtcipPort": 502,

"TimingPlanUpdateInterval_sec": 60,

"NtcipBackupTime_sec": 300,

"Vendor": "Econolite",

"TimingPlanMib": "/nojournal/bin/EconoliteMib.py",

"InactiveVehPhases":[],

"InactivePedPhases":[],

"SplitPhases":

{

"1": 6,

"3": 8,

"5": 2,

"7": 4

},

"PermissiveEnabled":

{

"1": true,

"3": true,

"5": true,

"7": true

}

}

}

For each intersection, HostIp, IntersectionName, MapPayload, IntersectionID, signal controller IpAddress, NtcipPort, and NtcipBackupTime_sec, Vendor, TimingPlanMib is required to be specified. The map payload can be obtained by creating an intersection map using USDOT map tool (https://webapp.connectedvcs.com/isd/). For simulation tools, MessageDistributorIP, PriorityRequestGeneratorServerIP is required to be specified.

The structure of mmitss-message-distributor-config.json is discussed in the section 5.1.

Step2: Define the docker files

It is required to specify two dockerfiles: 1. For MMITSS roadside software component Dockerfiles and 2. For simulation tools Dockerfile_simulation-tools. The dockerfiles are required to be placed in the mmitss/bin directory.

An example of the Dockerfile is as follows:

#---------------------------------------------------------------------------#

#    Dockerfile to build an x86 platform image for an intersection            #

#---------------------------------------------------------------------------#

FROM mmitssuarizona/mmitss-base-x86:1.3

COPY TrafficControllerInterface/x86/M_TrafficControllerInterface /mmitss

COPY SnmpEngine/x86/M_SnmpEngine /mmitss

COPY PriorityRequestSolver/x86/M_PriorityRequestSolver /mmitss

COPY PriorityRequestServer/x86/M_PriorityRequestServer /mmitss

COPY MapSpatBroadcaster/x86/M_MapSpatBroadcaster /mmitss

COPY supervisord.conf /mmitss

CMD ["/usr/bin/supervisord"]

An example of the Dockerfile_simulation-tools is as follows:

#---------------------------------------------------------------------------#

#    Dockerfile to build an x86 platform image for an intersection          #

#---------------------------------------------------------------------------#

FROM mmitssuarizona/mmitss-base-x86:1.3

COPY PriorityRequestGeneratorServer/x86/M_PriorityRequestGeneratorServer /mmitss

COPY MessageDistributor/x86/M_MessageDistributor /mmitss

RUN mkdir -p /mmitss/map

COPY supervisord_simulation-tools.conf /mmitss/supervisord.conf

CMD ["/usr/bin/supervisord"]

Step 3: Define supervisrod.conf and supervisor_simulation-tool.conf file

To monitor the applications in the container, it is required to define two supervisord.conf files. For MMITSS roadside software components supervisrod.conf and for simulation tools supervisrod-simulation-tools.conf. They has to be placed in the same level directory structure of Dockerfile (mmitss/bin).

Step 4: Define docker-compose.yml file

To build and run all the software components in the docker container, docker-compose.yml file has to be specified. The yml file has to be placed in the same level directory structure of Dockerfile (mmitss/bin).

- For each intersection, container name, source, and ipv4_address are required to be specified. Container name can be intersection name, source must be the directory of configuration files and ipv4 address has to match the Host IP address which is defined in the mmitss-phase3-master-config.json file.

- For simulations tools dockerfile name has to defined along with container name, source, ipv4_address. Container name can be simulation-tools, source must be the directory of configuration files and ipv4 address has to match the message-distributor IP address which is defined in the mmitss-phase3-master-config.json file.

- The ethernet interface of the computer and subnet has to be defined under the network. The ethernet interface of the computer that will be used to communicate with the containers can be obtained by using following command in a terminal on the computer:

ifconfig

An example of the docker-compose.yml file for two intersections and simulation tools is as follows:

version: "3.8"

services:

daisy-gavilan:

build:

context: ./

container_name: daisy-gavilan

volumes:

- type: bind

source: $MMITSS_ROOT/mmitss/bin/corridors/Anthem/Daisy-Gavilan/nojournal

target: /nojournal

networks:

mmitss_vlan:

ipv4_address: xxx.xxx.xxx.xxx

daisy-dedication:

build:

context: ./

container_name: daisy-dedication

volumes:

- type: bind

source: $MMITSS_ROOT/mmitss/bin/corridors/Anthem/Daisy-Dedication/nojournal

target: /nojournal

networks:

mmitss_vlan:

ipv4_address: xxx.xxx.xxx.yyy

simulation-tools:

build:

context: ./

dockerfile: Dockerfile_simulation-tools

container_name: simulation-tools

volumes:

- type: bind

source: $MMITSS_ROOT/mmitss/bin/corridors/simulation-tools/nojournal

target: /nojournal

networks:

mmitss_vlan:

ipv4_address: xxx.xxx.xxx.zzz

networks:

mmitss_vlan:

driver: macvlan

driver_opts:

parent: eno2

ipam:

config:

- subnet: xxx.xxx.xxx.0/24

Step 5: Build and run docker container

- Run the VISSIM simulation model

- Open a terminal in Linux box and directed to mmitss/scripts  directory

- Run following script to build all the software components:

mmitss_docker_make_all_for_x86.sh

- To build and run the docker containers go to mmitss/bin directory and execute the following command:

docker-compose up --build

- To monitor the containers execute the following command:

docker-compose exec <container_name> /bin/bash

It will direct inside the container. Following command can be executed to monitor the applications using supervisor.

supervisordctl

- To stop all the containers, execute the following command:

docker-compose stop

To stop a specific container, execute the following command

docker-compose stop <container_name>

- To start container the execute the following command:

docker-compose start

To start a specific container, execute the following command

docker-compose start <container_name>

- To down the whole macvlan network of docker containers, execute the following command

docker-compose down

- To up the whole macvlan network of docker containers, execute the following command

docker-compose up